Rear door cable adjustment
1. Begin by removing the lower rear door panel.

- Loosen the screw and nut 3/8” adjuster.

- Engage top latch by hand.

- Pull tension on cable until top latch opens.

- Slightly back tension off and retighten adjuster screw and nut (do not over tighten or cable will not function).

- Test door for function Open/close

- Once door is properly adjusted install a zip tie and pull loose slack out of cable.

*Installing a zip tie will greatly reduce the chances of failure in the future.

8. Reinstall door panel.